# Ways we could create prototype:

Mid-fi

Grab a large exercise ball or beach ball (~60–90cm — Kmart, Rebel, or a pool shop all stock these cheap). Mark 3–4 zones with tape, fabric patches, or stickers in different colors/textures so people can tell them apart by feel, not just sight — that also lets you test whether people notice zones without being told.

For the sound side, GarageBand's Live Loops grid (or Ableton if someone on the team has it) is genuinely built for exactly this: load a different stem like bass, drums, melody, pad, into each pad, and tapping a pad brings that layer in or out. One person sits off to the side, watches which zone gets touched, and taps the matching pad. For lights, the simplest fake is a phone flashlight or a cheap colour-changing smart bulb pointed at each zone, manually switched by the same person, no need to build anything.

That's a fully testable prototype with zero fabrication, which is the right call before you've validated the idea is even fun.

More final

Once WoZ testing confirms the concept works, a lightweight real build looks like: conductive fabric or copper tape patches (as touch sensors) wired to an MPR121 capacitive touch breakout board, connected to an Arduino or ESP32; the microcontroller then triggers a small addressable LED strip (NeoPixel/WS2812) near each zone and a DFPlayer Mini MP3 module + small speaker to play that zone's sound clip. Jaycar (you've got a few branches around Brisbane) stocks all of that — Arduino/ESP32 boards, NeoPixel strips, DFPlayer modules, conductive tape. If capacitive touch is finicky on a curved inflatable surface, a pressure pad (FSR) under a fabric patch is a more forgiving fallback — reacts to a press rather than needing skin contact.

Practically, it's easier to build the sensors and lights into a removable fabric "skin" (a stretchy cover with pockets sewn in for the electronics) that goes over the ball, rather than embedding anything directly into the inflatable itself — keeps it repairable and means the same skin could go on a bigger ball later for Week 12 without rebuilding the electronics.